# CHIVA Streaming Guidance — 30 June Scenario Test Report

# 1. Aim

This test report validates the real-time probe guidance output of the LLM Based Active Guidance System (Task 2). The system assists a vascular surgeon during duplex ultrasound examination by continuously interpreting the probe position and accumulated haemodynamic findings, then issuing a single imperative guidance instruction after each probe move or clip markings.

The test verifies that the AI pipeline:
  (a) directs the probe toward the correct anatomical zone at each stage of the examination.
  (b) correctly transitions from navigation (move) to maneuver instructions when the elimination test is required.
  (c) declares circuit completion (complete) at the precise moment all the findings for a shunt type is satisfied.

# 2. System Under Test

| Component | Details |
| --- | --- |
| Backend | Python 3.12 · Flask-SocketIO (threading mode) · port 7861 |
| Agent Framework | CrewAI sequential 5-agent pipeline (max_iter=3 per agent; 90 s crew timeout) |
| Text LLM | Groq · meta-llama/llama-3.3-70b-versatile · temperature 0.3 |
| Vision LLM (VLM) | Groq · meta-llama/llama-4-scout-17b-16e-instruct · timeout 15 s |
| Probe Localiser | Sliding-window classifier (window=10, stability=0.60) on posY ratio |
| LLM Threshold | New LLM call triggered when posY delta ≥ 0.06 OR anatomical region changes |
| VLM Threshold | New VLM call triggered when posY delta ≥ 0.05 |
| Clip Persistence | Session clips accumulate per leg; not cleared between probe moves |
| Transport | WebSocket (Socket.IO) — events: probe_move, clip_mark, stream_start, session_end |

# 3. Notation Reference

## 3a. Clip Types (EP / RP)

| Term | Definition |
| --- | --- |
| EP (Entry Point) | Blood exits the deep venous system into the superficial system. Confirmed by outward (away from deep vein and towards superficial veins) blood flow. |
| RP (Re-entry Point) | Blood returns from the superficial system back into the deep venous system. Confirmed by inward blood flow(away from the superficial veins and towards the deep veins i.e. normal blood flow), typically at a perforator or junction. |
| N1 | Deep vein (femoral, popliteal, or tibial). |
| N2 | Saphenous trunk — GSV (Great Saphenous Vein) or SSV (Small Saphenous Vein). Sits within the saphenous fascial compartment between the two fascia lines. |
| N3 | Superficial tributary — any named or unnamed branch above the fascial layer (e.g. Giacomini vein, anterior accessory saphenous, calf tributaries). |

## 3b. Guidance Actions

| Action | Meaning |
| --- | --- |
| move | Standard navigation instruction. The probe is not yet at the target zone or the circuit is not yet complete. Guidance is a single sentence with a direction word (distally / proximally / medially / posteriorly / laterally) and an anatomical target. Example: 'Move probe distally to mid-thigh GSV.' |
| maneuver | This action is outputted by the system when Elimination test is required. The system has detected the clip pattern that triggers the CHIVA elimination test (Rule 1: ep_n2_n3 + rp_n3_n1 + rp_n2_n1, with ep_n1_n2 present). The surgeon must compress the N3 tributary at the escape point and observe whether SSV/GSV trunk reflux persists or disappears. Guidance describes the compression/Doppler step — e.g. 'Compress tributary at escape point and record Doppler response.' Result 'No Reflux' → Type 3 (complete); Result 'Reflux' → Type 1+2 (complete). |
| complete | Circuit fully mapped. The minimum required findings for identifying a shunt type has been confirmed and all sufficiency questions (Q1 entry, Q2 trunk, Q3 escape, Q4 re-entry) are answered. No further probe movement is required for classification. Guidance is the fixed phrase: 'Circuit complete — classification confirmed, all zones mapped.' The system simultaneously emits a shunt_confirmed event with the type and evidence. |

# 4. Test Methodology

Each scenario is a synthetically generated sequence of WebSocket events that simulates a surgeon performing an assessment. Events are emitted sequentially with a 0.4 s inter-step delay. The test system waits up to 120 s for a guidance_update response before declaring a timeout.

Step types:
  PROBE_MOVE — emits probe_move with state parameters like region, leg, posY etc. Triggers VLM analysis (if posY delta ≥ 0.05 or region changed) and LLM guidance (if posY delta ≥ 0.06 or region changed); otherwise echoes the last computed guidance.
  CLIP_MARK — emits clip_mark with flow (EP/RP), from/to vessel types, posY, region, surface, and optional elimination_test result. Always forces a new LLM call.
  STREAM_START — creates/resets a session. No guidance expected; checked for session_ready.

Verdict rules:
  PASS  — all of: (1) action matches expected_action if set; (2) guidance contains at least one keyword from guidance_must_contain if set; (3) action is not the forbidden_action if set.
  FAIL  — any of the above conditions violated.
  SKIP  — step has no assertions (observe-only) or is stream_start.
  TIMEOUT — no guidance_update received within 120 s.

Leniency rule: if action='complete' or the word 'complete' appears in guidance at any point during a scenario, any step that failed solely because it expected action='complete' is automatically marked PASS. This accounts for the model reaching the correct terminal state one step earlier or later than the test script anticipated.

# 5. Literature Basis

| Reference | Relevance |
| --- | --- |
| Franceschi & Zamboni 2009 | Principles of Venous Hemodynamics. Nova Science Publishers. Chapter 9: CHIVA shunt type definitions (Types I–VI), circuit classification rules, elimination test protocol. Primary reference for all type definitions used in this test. |
| Gianesini 2014 | Gianesini S et al. Phlebology 2014; 29 Suppl 1: 5–23. Maneuver concordance rules: BOTH Paranà AND CR (compression-relaxation) must be positive for confirmed junctional incompetence at SFJ and SPJ. Single-maneuver positivity alone is insufficient for EP N1→N2 classification. |
| Delfrate 2023 | Delfrate R. JTAVR 2023; 8(2): 21–26. Standard duplex examination sequence; Valsalva/Paranà protocol at SFJ. |
| Adler 2022 | Adler G et al. RadioGraphics 2022; 42: 2184–2200. Lateral-decubitus positioning for SPJ assessment; augmentation criteria; anatomical variants of the saphenopopliteal junction. |

# 6. Results

## Overall Summary

| Scenario | Description | Passed | Failed | Timeouts |
| --- | --- | --- | --- | --- |
| spj_type1_plus2_reflux | SPJ entry (EP N1→N2 SPJ) → SSV escape (EP N2→N3) → RP N3→N1 → RP N2→N1 → Rule 1 maneuver → Reflux persists → Rule 3 comp… | 25 | 1 | 0 |
| type2b_dodd_perforator | Type 2B: EP N2→N2 (Dodd mid-thigh N2→N2 communicating vein, diam 4.1 mm) + RP N3→N1 calf. NO EP N1→N2; NO RP N2→N1. SFJ/… | 16 | 1 | 0 |
| type2c_no_elimination_needed | ADVERSARIAL — Type 2C: EP N2→N3 (Hunterian) + RP N3→N1 (calf) + RP N2→N1 (Hunterian) WITHOUT EP N1→N2. Crew must return … | 18 | 0 | 0 |
| type4_giacomini_circuit | Type 4: EP N1→N3 (femoral perf → Giacomini N3, posY=0.33 posterior) + RP N2→N1 (GSV trunk return path to deep, posY=0.09… | 14 | 2 | 0 |
| left_leg_type2b | LEFT leg Type 2B: EP N2→N2 (left Dodd, posY=0.26, diam 3.9 mm) + RP N3→N1 (left calf posY=0.70). Left SFJ/SPJ competent.… | 15 | 0 | 0 |
| left_leg_type5_looping | LEFT leg Type 5: EP N1→N3 (left Giacomini, posY=0.30) + RP N3→N2 (Giacomini-GSV junction posY=0.18) + EP N2→N3 (left Dod… | 21 | 1 | 0 |

OVERALL: FAILURES PRESENT

## Scenario: SPJ_TYPE1_PLUS2_REFLUX

SPJ entry (EP N1→N2 SPJ) → SSV escape (EP N2→N3) → RP N3→N1 → RP N2→N1 → Elimination Test Maneuver → Reflux persists → Findings complete (Type 1+2). Source: Franceschi 2009 p.114. 26 steps.

Checked: 26   |   Passed: 25   |   Failed: 1   |   Timeouts: 0

| # | Surgeon Action | Expected | LLM Guidance | Action | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | Start session (id: sA_spj_12rf) | session_ready event | -- | -- | SKIP |
| 2 | Move → SFJ [right leg] posY=0.05 anterior-medial | Action = "move" Guidance contains any of: SFJ, groin, femoral, junction, saphenofemoral | Move probe distally to SFJ | move | PASS |
| 3 | Move → SFJ [right leg] posY=0.07 anterior-medial | Action = "move" Must NOT = "complete" | Move probe distally to SFJ | move | PASS |
| 4 | Move → Upper Thigh [right leg] posY=0.12 medial | Action = "move" | Move probe distally to SFJ | move | PASS |
| 5 | Move → Hunterian [right leg] posY=0.43 medial | Action = "move" | Move probe distally to Hunterian perforators | move | PASS |
| 6 | Move → SPJ [right leg] posY=0.48 posterior | Action = "move" Guidance contains any of: popliteal, SPJ, posterior, junction | Move probe distally to Hunterian perforators | move | FAIL |
| x Guidance missing any of: popliteal, SPJ, posterior, junction |  |  |  |  |  |
| 7 | Move → SPJ [right leg] posY=0.51 posterior | Action = "move" | Move probe distally to SPJ | move | PASS |
| 8 | Move → SPJ [right leg] posY=0.55 posterior | Action = "move" | Move probe distally to SPJ | move | PASS |
| 9 | Mark: EP N1→N2 posY=0.51 SPJ right leg | Action = "move" Guidance contains any of: GSV, SSV, calf, posterior, trunk, distal | Move probe distally to GSV | move | PASS |
| 10 | Move → SSV [right leg] posY=0.59 posterior | Action = "move" | Move probe distally to GSV | move | PASS |
| 11 | Move → SSV [right leg] posY=0.63 posterior | Action = "move" | Move probe distally to GSV | move | PASS |
| 12 | Move → SSV [right leg] posY=0.66 posterior | Action = "move" | Move probe distally to SPJ | move | PASS |
| 13 | Mark: EP N2→N3 posY=0.65 SSV right leg | Action = "move" Must NOT = "complete" | Move probe distally to calf | move | PASS |
| 14 | Move → SSV [right leg] posY=0.73 posterior | Action = "move" | Move probe distally to SPJ | move | PASS |
| 15 | Move → Ankle [right leg] posY=0.90 posterior | Action = "move" | Move probe distally to GSV | move | PASS |
| 16 | Move → Ankle [right leg] posY=0.93 lateral | Action = "move" | Move probe distally to GSV | move | PASS |
| 17 | Mark: RP N3→N1 posY=0.91 Ankle right leg | Action = "move" Must NOT = "maneuver" | Move probe distally to GSV | move | PASS |
| 18 | Move → SSV [right leg] posY=0.79 posterior | Action = "move" | Move probe distally to popliteal | move | PASS |
| 19 | Move → SSV [right leg] posY=0.66 posterior | Action = "move" | Move probe distally to SPJ | move | PASS |
| 20 | Move → SSV [right leg] posY=0.60 posterior | Action = "move" | Move probe distally to medial calf perforators | move | PASS |
| 21 | Mark: RP N2→N1 posY=0.60 SSV right leg | Action = "maneuver" Guidance contains any of: compress, tributary, Doppler, record | Hold probe at escape site and compress tributary — record Doppler response | maneuver | PASS |
| 22 | Move → SSV [right leg] posY=0.65 posterior | Action = "maneuver" Guidance contains any of: compress, tributary | Hold probe at escape site and compress tributary — record Doppler response | maneuver | PASS |
| 23 | Move → SSV [right leg] posY=0.61 posterior | Action = "maneuver" | Hold probe at escape site and compress tributary — record Doppler response | maneuver | PASS |
| 24 | Mark: EP N2→N3 posY=0.65 SSV right leg Elim: Reflux | Action = "complete" Guidance contains any of: circuit, classification, complete, mapped | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 25 | Move → SPJ [right leg] posY=0.51 posterior | Action = "complete" Guidance contains any of: circuit, classification | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 26 | Move → SSV [right leg] posY=0.65 posterior | Action = "complete" | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 27 | Move → SFJ [right leg] posY=0.06 anterior-medial | Action = "complete" | Circuit complete — classification confirmed, all zones mapped | complete | PASS |

## Scenario: TYPE2B_DODD_PERFORATOR

Type 2B: EP N2→N2 (Dodd mid-thigh N2→N2 vein) + RP N3→N1 calf. NO EP N1→N2; NO RP N2→N1. SFJ/SPJ both competent. First Type 2B scenario (not in v1/v2). Source: Franceschi 2009 p.118, AVF/Delfrate 2023 perf criteria. 17 steps.

Checked: 17   |   Passed: 16   |   Failed: 1   |   Timeouts: 0

| # | Surgeon Action | Expected | LLM Guidance | Action | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | Start session (id: sB_2b_dodd) | session_ready event | -- | -- | SKIP |
| 2 | Move → SFJ [right leg] posY=0.05 anterior-medial | Action = "move" Guidance contains any of: SFJ, groin, femoral, junction | Move probe distally to SFJ | move | PASS |
| 3 | Move → SFJ [right leg] posY=0.07 anterior-medial | Action = "move" Must NOT = "complete" | Move probe distally to SFJ | move | PASS |
| 4 | Move → Upper Thigh [right leg] posY=0.11 medial | Action = "move" | Move probe distally to SFJ | move | PASS |
| 5 | Move → Dodd [right leg] posY=0.22 medial | Action = "move" Guidance contains any of: Dodd, thigh, perforator, mid | Move probe distally to mid-thigh GSV | move | PASS |
| 6 | Move → Dodd [right leg] posY=0.27 medial | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 7 | Move → Dodd [right leg] posY=0.29 medial | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 8 | Mark: EP N2→N2 posY=0.28 Dodd right leg | Action = "move" Guidance contains any of: distal, calf, tributary, escape | Move probe distally to GSV | move | PASS |
| 9 | Move → Hunterian [right leg] posY=0.39 medial | Action = "move" Must NOT = "complete" | Move probe distally to GSV | move | PASS |
| 10 | Move → SPJ [right leg] posY=0.49 posterior | Action = "move" Guidance contains any of: popliteal, SPJ, posterior | Move probe distally to GSV | move | FAIL |
| x Guidance missing any of: popliteal, SPJ, posterior |  |  |  |  |  |
| 11 | Move → SPJ [right leg] posY=0.53 posterior | Action = "move" | Move probe distally to GSV | move | PASS |
| 12 | Move → Calf [right leg] posY=0.62 medial | Action = "move" | Move probe distally to GSV | move | PASS |
| 13 | Move → Calf [right leg] posY=0.68 medial | Action = "move" | Move probe distally to GSV | move | PASS |
| 14 | Move → Calf [right leg] posY=0.73 medial | Action = "move" | Move probe distally to GSV | move | PASS |
| 15 | Mark: RP N3→N1 posY=0.71 Calf right leg | Action = "complete" Guidance contains any of: circuit, classification, complete, mapped | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 16 | Move → Dodd [right leg] posY=0.28 medial | Action = "complete" Guidance contains any of: circuit, classification | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 17 | Move → SFJ [right leg] posY=0.06 anterior-medial | Action = "complete" | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 18 | Move → SPJ [right leg] posY=0.51 posterior | Action = "complete" | Circuit complete — classification confirmed, all zones mapped | complete | PASS |

## Scenario: TYPE2C_NO_ELIMINATION_NEEDED

ADVERSARIAL — Type 2C: EP N2→N3 (Hunterian) + RP N3→N1 (calf) + RP N2→N1 (Hunterian) WITHOUT EP N1→N2. System must return complete (Type 2C), NOT maneuver. Source: Franceschi 2009 p.119, Type IIc. 18 steps.

Checked: 18   |   Passed: 18   |   Failed: 0   |   Timeouts: 0

| # | Surgeon Action | Expected | LLM Guidance | Action | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | Start session (id: sC_2c_noel) | session_ready event | -- | -- | SKIP |
| 2 | Move → SFJ [right leg] posY=0.05 anterior-medial | Action = "move" Guidance contains any of: SFJ, groin, femoral, junction | Move probe distally to SFJ | move | PASS |
| 3 | Move → SFJ [right leg] posY=0.07 anterior-medial | Action = "move" | Move probe distally to SFJ | move | PASS |
| 4 | Move → Upper Thigh [right leg] posY=0.14 medial | Action = "move" | Move probe distally to SFJ | move | PASS |
| 5 | Move → Dodd [right leg] posY=0.25 medial | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 6 | Move → Hunterian [right leg] posY=0.37 medial | Action = "move" | Move probe distally to Hunterian perforators | move | PASS |
| 7 | Move → Hunterian [right leg] posY=0.41 medial | Action = "move" | Move probe distally to Hunterian perforators | move | PASS |
| 8 | Mark: EP N2→N3 posY=0.41 Hunterian right leg | Action = "move" Must NOT = "complete" | Move probe distally to Hunterian perforators | move | PASS |
| 9 | Move → SPJ [right leg] posY=0.49 posterior | Action = "move" Guidance contains any of: popliteal, SPJ, posterior | Move probe distally to SPJ | move | PASS |
| 10 | Move → SPJ [right leg] posY=0.53 posterior | Action = "move" | Move probe distally to SPJ | move | PASS |
| 11 | Move → Calf [right leg] posY=0.60 medial | Action = "move" | Move probe distally to posterior tibial perforator | move | PASS |
| 12 | Move → Calf [right leg] posY=0.67 medial | Action = "move" | Move probe distally to posterior tibial perforator | move | PASS |
| 13 | Move → Calf [right leg] posY=0.73 medial | Action = "move" | Move probe distally to posterior tibial perforator | move | PASS |
| 14 | Mark: RP N3→N1 posY=0.71 Calf right leg | Action = "move" Must NOT = "maneuver" | Move probe distally to posterior tibial perforator | move | PASS |
| 15 | Move → Calf [right leg] posY=0.60 medial | Action = "move" | Move probe distally to posterior tibial perforator | move | PASS |
| 16 | Move → Hunterian [right leg] posY=0.45 medial | Action = "move" | Move probe distally to Hunterian perforators | move | PASS |
| 17 | Mark: RP N2→N1 posY=0.47 Hunterian right leg | Action = "complete" Must NOT = "maneuver" Guidance contains any of: circuit, classification, complete, mapped | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 18 | Move → Hunterian [right leg] posY=0.41 medial | Action = "complete" Guidance contains any of: circuit, classification | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 19 | Move → SFJ [right leg] posY=0.06 anterior-medial | Action = "complete" | Circuit complete — classification confirmed, all zones mapped | complete | PASS |

## Scenario: TYPE4_GIACOMINI_CIRCUIT

Type 4: EP N1→N3 (femoral perf → Giacomini N3, posY=0.33 posterior) + RP N2→N1 (GSV trunk return path to deep, posY=0.09). Trunk is in the RETURN path only. Distinct from Type VI (no RP N3→N1 here; N2 involved in return). Source: Franceschi 2009 p.119. 16 steps.

Checked: 16   |   Passed: 14   |   Failed: 2   |   Timeouts: 0

| # | Surgeon Action | Expected | LLM Guidance | Action | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | Start session (id: sD_t4_giac) | session_ready event | -- | -- | SKIP |
| 2 | Move → SFJ [right leg] posY=0.05 anterior-medial | Action = "move" Guidance contains any of: SFJ, groin, femoral, junction, valve, terminal | Move probe distally to SFJ | move | PASS |
| 3 | Move → SFJ [right leg] posY=0.07 anterior-medial | Action = "move" | Move probe distally to SFJ | move | PASS |
| 4 | Move → Upper Thigh [right leg] posY=0.11 medial | Action = "move" | Move probe distally to SFJ | move | PASS |
| 5 | Move → Giacomini [right leg] posY=0.10 posterior | Action = "move" Guidance contains any of: posterior, thigh, Giacomini, tributary | Move probe distally to SFJ | move | FAIL |
| x Guidance missing any of: posterior, thigh, Giacomini, tributary |  |  |  |  |  |
| 6 | Move → Giacomini [right leg] posY=0.18 posterior | Action = "move" | Move probe distally to SFJ | move | PASS |
| 7 | Move → Giacomini [right leg] posY=0.24 posterior | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 8 | Move → Giacomini [right leg] posY=0.31 posterior | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 9 | Move → Giacomini [right leg] posY=0.33 posterior | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 10 | Mark: EP N1→N3 posY=0.33 Giacomini right leg | Action = "move" Must NOT = "complete" Guidance contains any of: trunk, GSV, thigh, medial, proximal | Move probe distally to calf | move | FAIL |
| x Guidance missing any of: trunk, GSV, thigh, medial, proximal |  |  |  |  |  |
| 11 | Move → Giacomini [right leg] posY=0.15 posterior | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 12 | Move → Upper Thigh [right leg] posY=0.12 medial | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 13 | Move → Upper Thigh [right leg] posY=0.09 medial | Action = "move" | Move probe distally to calf | move | PASS |
| 14 | Mark: RP N2→N1 posY=0.09 Upper Thigh right leg | Action = "complete" Guidance contains any of: circuit, classification, complete, mapped | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 15 | Move → Giacomini [right leg] posY=0.33 posterior | Action = "complete" Guidance contains any of: circuit, classification | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 16 | Move → Upper Thigh [right leg] posY=0.09 medial | Action = "complete" | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 17 | Move → SFJ [right leg] posY=0.06 anterior-medial | Action = "complete" | Circuit complete — classification confirmed, all zones mapped | complete | PASS |

## Scenario: LEFT_LEG_TYPE2B

LEFT leg Type 2B: EP N2→N2 (left Dodd, posY=0.26, diam 3.9 mm) + RP N3→N1 (left calf posY=0.70). Left SFJ/SPJ competent. Tests left-leg coordinate routing AND Type 2B on left. Neither left-leg nor Type 2B combination tested in v1/v2. 15 steps.

Checked: 15   |   Passed: 15   |   Failed: 0   |   Timeouts: 0

| # | Surgeon Action | Expected | LLM Guidance | Action | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | Start session (id: sE_ll_2b) | session_ready event | -- | -- | SKIP |
| 2 | Move → SFJ [left leg] posY=0.05 anterior-medial | Action = "move" Guidance contains any of: SFJ, groin, femoral, junction | Move probe distally to SFJ | move | PASS |
| 3 | Move → SFJ [left leg] posY=0.07 anterior-medial | Action = "move" Must NOT = "complete" | Move probe distally to SFJ | move | PASS |
| 4 | Move → Upper Thigh [left leg] posY=0.12 medial | Action = "move" | Move probe distally to SFJ | move | PASS |
| 5 | Move → Dodd [left leg] posY=0.22 medial | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 6 | Move → Dodd [left leg] posY=0.26 medial | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 7 | Mark: EP N2→N2 posY=0.26 Dodd left leg | Action = "move" Guidance contains any of: calf, distal, tributary, left | Move probe distally to GSV | move | PASS |
| 8 | Move → Hunterian [left leg] posY=0.40 medial | Action = "move" | Move probe distally to GSV | move | PASS |
| 9 | Move → SPJ [left leg] posY=0.49 posterior | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 10 | Move → SPJ [left leg] posY=0.52 posterior | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 11 | Move → Calf [left leg] posY=0.61 medial | Action = "move" | Move probe distally to GSV | move | PASS |
| 12 | Move → Calf [left leg] posY=0.67 medial | Action = "move" | Move probe distally to paratibial perforator | move | PASS |
| 13 | Move → Calf [left leg] posY=0.72 medial | Action = "move" | Move probe distally to paratibial perforator | move | PASS |
| 14 | Mark: RP N3→N1 posY=0.70 Calf left leg | Action = "complete" Guidance contains any of: circuit, classification, complete, mapped | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 15 | Move → Dodd [left leg] posY=0.26 medial | Action = "complete" Guidance contains any of: circuit, classification | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 16 | Move → SFJ [left leg] posY=0.06 anterior-medial | Action = "complete" | Circuit complete — classification confirmed, all zones mapped | complete | PASS |

## Scenario: LEFT_LEG_TYPE5_LOOPING

LEFT leg Type 5: EP N1→N3 (left Giacomini, posY=0.30) + RP N3→N2 (Giacomini-GSV junction posY=0.18) + EP N2→N3 (left Dodd escape, posY=0.28) + RP N3→N1 (left calf, posY=0.70). NO RP N2→N1. Source: Franceschi 2009 p.121. 22 steps.

Checked: 22   |   Passed: 21   |   Failed: 1   |   Timeouts: 0

| # | Surgeon Action | Expected | LLM Guidance | Action | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | Start session (id: sF_ll_t5) | session_ready event | -- | -- | SKIP |
| 2 | Move → SFJ [left leg] posY=0.05 anterior-medial | Action = "move" Guidance contains any of: SFJ, groin, femoral, junction | Move probe distally to SFJ | move | PASS |
| 3 | Move → SFJ [left leg] posY=0.07 anterior-medial | Action = "move" | Move probe distally to SFJ | move | PASS |
| 4 | Move → Upper Thigh [left leg] posY=0.12 medial | Action = "move" | Move probe proximally to SFJ | move | PASS |
| 5 | Move → SPJ [left leg] posY=0.49 posterior | Action = "move" Guidance contains any of: popliteal, SPJ, posterior | Move probe distally to SPJ | move | PASS |
| 6 | Move → SPJ [left leg] posY=0.52 posterior | Action = "move" | Move probe distally to SPJ | move | PASS |
| 7 | Move → Giacomini [left leg] posY=0.14 posterior | Action = "move" Guidance contains any of: posterior, thigh, Giacomini, tributary | Move probe distally to SFJ | move | FAIL |
| x Guidance missing any of: posterior, thigh, Giacomini, tributary |  |  |  |  |  |
| 8 | Move → Giacomini [left leg] posY=0.22 posterior | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 9 | Move → Giacomini [left leg] posY=0.30 posterior | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 10 | Mark: EP N1→N3 posY=0.30 Giacomini left leg | Action = "move" Must NOT = "complete" Guidance contains any of: GSV, trunk, thigh, proximal, medial | Move probe distally to mid-thigh GSV | move | PASS |
| 11 | Move → Giacomini [left leg] posY=0.18 posterior | Action = "move" | Move probe distally to calf | move | PASS |
| 12 | Mark: RP N3→N2 posY=0.18 Giacomini left leg | Action = "move" Must NOT = "complete" | Move probe distally to mid-thigh GSV | move | PASS |
| 13 | Move → Upper Thigh [left leg] posY=0.17 medial | Action = "move" | Move probe distally to mid-thigh GSV | move | PASS |
| 14 | Move → Dodd [left leg] posY=0.24 medial | Action = "move" | Move probe distally to calf | move | PASS |
| 15 | Move → Dodd [left leg] posY=0.28 medial | Action = "move" | Move probe distally to calf | move | PASS |
| 16 | Mark: EP N2→N3 posY=0.28 Dodd left leg | Action = "move" Must NOT = "complete" | Move probe distally to mid-thigh GSV | move | PASS |
| 17 | Move → Hunterian [left leg] posY=0.40 medial | Action = "move" | Move probe distally to Hunterian perforators | move | PASS |
| 18 | Move → Calf [left leg] posY=0.60 medial | Action = "move" | Move probe distally to perforator | move | PASS |
| 19 | Move → Calf [left leg] posY=0.68 medial | Action = "move" | Move probe distally to paratibial perforator | move | PASS |
| 20 | Mark: RP N3→N1 posY=0.70 Calf left leg | Action = "complete" Guidance contains any of: circuit, classification, complete, mapped | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 21 | Move → Giacomini [left leg] posY=0.30 posterior | Action = "complete" Guidance contains any of: circuit, classification | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 22 | Move → Dodd [left leg] posY=0.28 medial | Action = "complete" | Circuit complete — classification confirmed, all zones mapped | complete | PASS |
| 23 | Move → SFJ [left leg] posY=0.06 anterior-medial | Action = "complete" | Circuit complete — classification confirmed, all zones mapped | complete | PASS |