Shunt Analysis

Shunt Type 1:

Shunt type 3 variant 1:

Shunt type 1+2 with small diameter :

note: shunt type 1+2 has the same markers as shunt type 3, only difference is the RP N3->N1 marker has elimination test result as "Reflux" instead of "No Reflux".

Shunt type 2 - equal calibre, unequal distance :

Shunt type 2 - equal calibre, yes to drainage :

Shunt type 2 - unequal calibre, no drainage :

Current Rule Based Code being used for Shunt Classification (Task-1) :

import logging

from typing import Any

class ShuntManager:

"""

Manages ...

"""

def __init__(self) -> None:

self.logger = logging.getLogger(__name__)

def classify_shunt(self, clip_list: list[dict[str, Any]]) -> dict:

"""Takes clipList from QML and returns:

{

"st1": bool,

"st3": bool,

"st12": bool,

"needs_elim_test": bool,

...

}

"""

st1 = False

st3 = False

st12 = False

st2a = False

st2b = False

st2c = False

st_detected = True

ask_diameter = False

ask_aplastic = False

ask_reflux_perforator = False

ask_branching = False

ep12 = False

ep23 = False

ep22 = False

rp32 = False

rp31 = False

rp21 = False

rp3 = False

many_rp3 = False  # more than 1 RP at tributary

many_rp21 = False

ep12_at_sfj = False

ep12_at_hunterian = False

elim_test_done = False

needs_elim_test = False

reflux_persists = False

sfj_min_x = 0.118

sfj_max_x = 0.473

sfj_min_y = 0.0

sfj_max_y = 0.098

hunterian_min_x = 0.118

hunterian_max_x = 0.473

hunterian_min_y = 0.098

hunterian_max_y = 0.353

reasoning = []

ligation = []

# list of ep12 to ligate

ep12_indexes = []

# list of rp21 to ligate, compare y position

rp21_indexes = []

# list of ep23 to ligate

ep23_indexes = []

# list of ep23 and ep22 to ligate for shunt type 2, compare y position

ep23_y_indexes = []

# Reset all ligation rectangles first in QML list

for clip in clip_list:

if "ep_ligation_rect" in clip:

del clip["ep_ligation_rect"]

if "ep_ligation_rect2" in clip:

del clip["ep_ligation_rect2"]

if "rp_ligation_rect" in clip:

del clip["rp_ligation_rect"]

# ==========================================================

# === 1. PRE-SHUNT TYPE ASSESSMENT CLIPLIST CRITERIAS CHECK ===

# ==========================================================

for idx, clip in enumerate(clip_list):

x = clip.get("posXRatio")

y = clip.get("posYRatio")

flow = clip.get("flow")

from_type = clip.get("fromType")

to_type = clip.get("toType")

elim_test_result = clip.get("eliminationTest")

# Check for EP at N1->N2

if flow == "EP" and from_type == "N1" and to_type == "N2":

ep12 = True

ep12_indexes.append(idx)

# Check if there is EP N1->N2 at SFJ or Hunterian Perforator

if (

x is not None

and y is not None

and hunterian_min_x <= x <= hunterian_max_x

and hunterian_min_y <= y <= hunterian_max_y

):

ep12_at_hunterian = True

ep12_at_sfj = False

elif (

x is not None

and y is not None

and sfj_min_x <= x <= sfj_max_x

and sfj_min_y <= y <= sfj_max_y

):

ep12_at_sfj = True

ep12_at_hunterian = False

# Check for EP at N2->N3

if flow == "EP" and from_type == "N2" and to_type == "N3":

ep23 = True

ep23_indexes.append(idx)

ep23_y_indexes.append((idx, y))

# Check if elim test has been done

if elim_test_result != "":

elim_test_done = True

if elim_test_result == "Reflux":

reflux_persists = True

elif elim_test_result == "No Reflux":

reflux_persists = False

# Check for EP at N2->N2

if flow == "EP" and from_type == "N2" and to_type == "N2":

ep22 = True

ep23_y_indexes.append((idx, y))

# Check for RP at N3->N2

if flow == "RP" and from_type == "N3" and to_type == "N2":

rp32 = True

# Check for RP at N3->N1

if flow == "RP" and from_type == "N3" and to_type == "N1":

rp31 = True

# Check for more than one RP at tributary

if flow == "RP" and from_type == "N3":

if rp3:

many_rp3 = True

else:

rp3 = True

# Check for RP at N2->N1

if flow == "RP" and from_type == "N2" and to_type == "N1":

rp21_indexes.append(

(idx, y)

)  # include y-axis position to ensure most distal RP does not get ligated

if rp21:

many_rp21 = True

else:

rp21 = True

# Check elim test result at EP N2->N3

if ep23 and elim_test_done:

if elim_test_result == "Reflux":

reflux_persists = True

elif elim_test_result == "No Reflux":

reflux_persists = False

# ==========================================================

# 2. SHUNT TYPE DECISION TREE

# ==========================================================

""" Determine shunt types based on collected data above """

if ep12_at_sfj or ep12_at_hunterian:

if ep12_at_sfj:

reasoning.append("• EP exists from N1 to N2 at SFJ \n")

if ep12_at_hunterian:

reasoning.append("• EP exists from N1 to N2 at Hunterian perforator \n")

# --- SHUNT TYPE 1 ---

if not ep23:

reasoning.append("• No EP exists from N2 to N3 \n")

if rp21 and not (rp31 or rp32):

st1 = True

reasoning.append(

"• RP exists along N2, but no RP exists along N3 \n"

)

else:

st_detected = False

reasoning = []

# EP at N2->N3, and RP exists at N3 only, or RP exists at N2 only through N3->N2

elif ep23:

reasoning.append("• EP exists from N2 to N3 \n")

# --- SHUNT TYPE 3 ---

if (rp32 or rp31) and (not rp21):

st3 = True

reasoning.append(

"• RP exists along N3, but no RP exists along N2 \n"

)

elif rp32 and rp21:

st3 = True

reasoning.append("• RP exists along N2, and through N3 to N2 \n")

# --- Elimination Test needed to tell between Type 3 and 1+2 ---

# Requires thinking of how to add order between second type of Type 3 (2 RPs)

elif rp31 and rp21:

reasoning.append(

"• RP exists along both N2 and N3, and towards N1 \n"

)

if not elim_test_done:

needs_elim_test = True

reasoning.append("• Elimination test missing. \n")

ligation.append("• Elimination test missing. \n")

# --- SHUNT TYPE 1+2 ---

elif reflux_persists:

st12 = True

reasoning.append("• Reflux persists in elimination test \n")

if not many_rp21 and many_rp3:

reasoning.append(

"• There is one RP at N2 and more than one RP at tributary\n"

)

elif many_rp21 and many_rp3:

reasoning.append(

"• There is more than one RP at N2 and more than one RP at tributary\n"

)

# --- SHUNT TYPE 3 ---

elif not reflux_persists:

st3 = True

reasoning.append(

"• Reflux does not persist in elimination test \n"

)

else:  # failsafe branch for unhandled logic

st_detected = False  # type: ignore[unreachable]

reasoning = []

# fallback incase no shunt type detected but entered this branch

else:

st_detected = False

reasoning = []

# --- SHUNT TYPE 2A ---

elif ep23 and not ep12:

st2a = True

reasoning.append(

"• EP exists from N2 to N3, but no EP exists from N1 to N2 \n"

)

elif ep22 and not ep12:

reasoning.append("• EP starts at mid GSV from N2 to N2 \n")

# --- SHUNT TYPE 2B ---

if (rp32 or rp31) and (not rp21):

st2b = True

reasoning.append("• RP exists along N3, but no RP exists along N2 \n")

# --- SHUNT TYPE 2C ---

elif (rp32 or rp31) and rp21:

st2c = True

reasoning.append("• RP exists along both N2 (GSV) and N3 \n")

else:

st_detected = False

reasoning.append(

"• But neither Shunt Type 2B nor 2C because RP does not exist along N3 only or along both N2 and N3 \n"

)

# If none of the shunt types are detected, clear reasoning

else:

st_detected = False

reasoning = []

# ==========================================================

# === 3. LIGATION DECISION TREE ===

# ==========================================================

""" Append ligation text and assign rectangle markers to EP/RP that should be ligated """

# Ligation for shunt type 1

if st1:

if ep12_at_sfj:

ligation.append("• Ligation at the SFJ \n")

elif ep12_at_hunterian:

ligation.append(

"• Ligation at the Hunterian Perforator closer to N2 \n"

)

if many_rp21:

ligation.append(

"• When appropriate, ligate below each RP along N2, except the most distal RP \n"

)

# Sort list of RP from N2 according to y-axis position, then remove the last one (most distal)

self._ligate_all_rp21_except_distal(clip_list, rp21_indexes)

# Update clips with field for rectangle markings (EP12)

for idx in ep12_indexes:

self._mark_ligation_rect(clip_list, idx, "ep_ligation_rect")

# Ligation for shunt type 1+2

if st12:

ligation = []

ligation.append(

"• Shunt Type 1+2 detected, requires more data to classify ligation"

)

if not many_rp21 and rp3:

ask_diameter = True

elif many_rp21 and rp3:

ligation = []

ligation.append(

"• Ligate SFJ or Hunterian perforator, and every refluxing tributary\n "

"• When appropriate, ligate below each RP along N2, except the most distal RP\n "

"• If one of the RP at refluxing tributary is close to the N2, and that special RP is dilated, ligate that special RP."

)

# Add rectangles to ligate all RP21 except most distal

self._ligate_all_rp21_except_distal(clip_list, rp21_indexes)

# Update clips with field for rectangle markings (EP23)

for idx in ep23_indexes:

self._mark_ligation_rect(clip_list, idx, "ep_ligation_rect2")

# Update clips with field for rectangle markings (EP12)

for idx in ep12_indexes:

self._mark_ligation_rect(clip_list, idx, "ep_ligation_rect")

# Not yet implemented: Special case where one of the RP32 is close to N2 (Fig 6.28d)

# Ligation for shunt type 3

if st3:

ligation = []

ask_aplastic = True  # Not yet implemented: Aplastic segment

if not many_rp3:

ligation.append("• Ligate the EP at N2->N3\n")

elif many_rp3:

ligation.append(

"• Ligate every refluxing tributary where it joins the N2 (first step of CHIVA 2)\n"

)

if ep12_at_sfj:

ligation.append(

"• Follow up from 6 weeks to 6/12 months to assess reflux at proximal N2"

"\n• If there is reflux, the SFJ must be treated/ligated\n"

)

if ep12_at_hunterian:

ask_reflux_perforator = True

# Update clips with field for rectangle markings (EP23)

for idx in ep23_indexes:

self._mark_ligation_rect(clip_list, idx, "ep_ligation_rect2")

# Not yet implemented: Special case combined with Shunt Type 1 for now (Fig. 6.33)

# Ligation for shunt type 2

if st2a or st2b or st2c:

ligation.append("• Ligate highest EP at N2->N3\n")

if many_rp3:

ligation.append(

"• In case there are more branching along N3, ligate the tributary based on calibre, distance to perforator, and drainage\n"

)

ask_branching = True

# Mark highest EP23 with rectangle

self._ligate_highest_ep23(clip_list, ep23_y_indexes)

# Ligation for no shunt types detected

if not st_detected:

ligation = []

return {

"ST1": st1,

"ST3": st3,

"ST12": st12,

"ST2A": st2a,

"ST2B": st2b,

"ST2C": st2c,

"shuntDetected": st_detected,

"elimTestDone": elim_test_done,

"needsElimTest": needs_elim_test,

"refluxPersists": reflux_persists,

"EP12AtSFJ": ep12_at_sfj,

"EP1AtHunterian": ep12_at_hunterian,

"EP23": ep23,

"RP32": rp32,

"RP31": rp31,

"RP21": rp21,

"reasoning": reasoning,

"ligation": ligation,

"updatedClipList": clip_list,

"askDiameter": ask_diameter,

"askAplastic": ask_aplastic,

"askRefluxPerforator": ask_reflux_perforator,

"askBranching": ask_branching,

}

def _ligate_all_rp21_except_distal(self, clip_list: list[dict[str, Any]], rp21_indexes: list) -> None:

# Add rectangles to ligate all RP21 except most distal

rp21_sorted = sorted(rp21_indexes, key=lambda t: t[1])

rp21_to_ligate = rp21_sorted[:-1]  # all except last RP

for idx, _ in rp21_to_ligate:

self._mark_ligation_rect(clip_list, idx, "rp_ligation_rect")

def _ligate_highest_ep23(self, clip_list: list[dict[str, Any]], ep23_y_indexes: list) -> None:

# For shunt type 2, add rectangle to ligate an EP23 or EP22 based on highest y position

ep23_sorted = sorted(ep23_y_indexes, key=lambda t: t[1])

highest_ep_idx = ep23_sorted[0][0]  # all except last RP

self._mark_ligation_rect(clip_list, highest_ep_idx, "ep_ligation_rect2")

def _mark_ligation_rect(self, clip_list: list[dict[str, Any]], index: int, field_name: str) -> None:

# Update clips with field for rectangle markings (e.g. EP23)

original = clip_list[index]

updated = original.copy()

updated[field_name] = True

clip_list[index] = updated