# Cygnus Med – Task-2 Active Guidance System

# LLM-Evaluated Clinical Test Report: Type 1+2 Shunt (Real Patient Case)

Test date:  2026-07-14 17:08
Source video:  Media - Shunt type 1.mp4  (Mindray M9CV, 30 fps, 3 min 4 s)
Diagnosis:  Type 1+2 combined shunt — EP N1→N2 SFJ + RP N2→N1 thigh + EP N2→N3 mid-calf (Reflux) + RP N3→N1 lower calf
Evaluation:  LLM-based (Groq llama-3.3-70b-versatile) — no keyword lists
Steps:  22 examination states across 8 clinical phases

## 1. Summary Scoreboard

| Metric | Score | Verdict |
| --- | --- | --- |
| Guidance CORRECT (score 3) | 5/22 (23%) | NEEDS IMPROVEMENT |
| Guidance PARTIAL (score 2) | 1/22 |  |
| Guidance WRONG (score 1) | 16/22 | INVESTIGATE |
| NO RESPONSE (score 0) | 0/22 | PASS |
| Action flag correct | 18/22 | FAIL (4 wrong flags) |
| Avg crew latency | 9404 ms | < 15 s target |

## 2. Step-by-Step Evaluation

| Step (time) | Phase | Surgeon Action (Ground Truth Event) | Probe Position | Clips Count | System Guidance | Action Flag | Expected Action | Action OK? | Eval Score | LLM Evaluator Reasoning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| #1 0–5 s | Ph1 SFJ Arrival | Probe first placed at anteromedial groin crease. B-mode shows two adjacent anechoic ovals: larger CFV (N1) and smaller GSV (N2) on the same depth plane at the junction. | region=SFJ posY=0.06 surface=anterior-medial | 0 | Move proximally toward GSV in saphenous compartment at upper thigh | move | move | YES | WRONG (1/3) | The AI guidance directs the probe away from the groin, instructing to move proximally toward the GSV in the saphenous compartment at the upper thigh, which is incorrect as the probe has just arrived at the SFJ and needs to confirm the CFV-GSV relationship and prepare for further maneuvers, not move away from the area of interest. |
| #2 5–15 s | Ph1 SFJ Orientation | Probe rotated to longitudinal view then back to transverse. Terminal valve identified just distal to SFJ. Probe centred exactly on the saphenofemoral junction. | region=SFJ posY=0.07 surface=anterior-medial | 0 | Move distally toward GSV in saphenous compartment at upper thigh | move | move | YES | WRONG (1/3) | The AI system's guidance to move distally toward the GSV is premature and misses the crucial step of assessing terminal valve competence through Valsalva or Paranà maneuver at the current position, which is essential for EP assessment at SFJ. The correct action should be to instruct the surgeon to perform a maneuver to test valve competence, not to move the probe. |
| #3 15–30 s | Ph1 SFJ Maneuvers | Surgeon performs Valsalva and Paranà maneuvers at SFJ. Both tests positive — Doppler shows reversal of flow from CFV into GSV on both maneuvers. SFJ incompetence confirmed. | region=SFJ posY=0.06 surface=anterior-medial | 0 | Move distally toward SPJ junction in popliteal fossa | move | move | YES | WRONG (1/3) | The AI system's guidance to move distally toward the SPJ junction in the popliteal fossa is incorrect because it misses the most important clinical need at this step, which is to mark the finding of SFJ incompetence or direct the probe to start mapping the GSV trunk reflux, given that both Valsalva and Paranà maneuvers are positive. |
| #4 30–42 s | Ph1 EP N1→N2 Marked | Surgeon marks EP N1→N2 at SFJ (posY 0.06). Clip confirmed. SFJ entry point for shunt circuit now established. Q1 answered. Probe remains at groin for a moment before moving distally. | region=SFJ posY=0.06 surface=anterior-medial | 1 | Move distally toward GSV in upper thigh | move | move | YES | CORRECT (3/3) | The AI system's guidance to move distally toward the GSV in the upper thigh aligns with the clinical intent to track the GSV and assess for trunk reflux, directly addressing the now open question Q2. |
| #5 42–52 s | Ph2 Proximal Thigh | Probe moved to proximal anteromedial thigh, just below groin crease. GSV oval visible within the saphenous fascial compartment. No perforator identified at this level. | region=Upper Thigh posY=0.14 surface=anterior-medial | 1 | Move distally toward GSV in saphenous compartment at upper thigh | move | move | YES | WRONG (1/3) | The AI guidance instructs the surgeon to move distally toward the GSV, but it fails to mention the application of Paranà (augmentation) to detect trunk reflux or to continue scanning toward the Hunterian zone, which is the correct clinical intent at this step. |
| #6 52–62 s | Ph2 Hunterian Zone | Probe at medial mid-thigh (Hunterian canal, proximal). GSV still prominent in fascial compartment. Hunterian perforator junction sought but not yet identified. | region=Hunterian posY=0.26 surface=anterior-medial | 1 | Move medially toward Hunterian perforator on medial proximal thigh | move | move | YES | WRONG (1/3) | The AI system's guidance to move medially toward the Hunterian perforator is incorrect because the primary goal at this step is to confirm trunk reflux, not to locate the perforator, and the system should have directed the surgeon to apply Paranà at this level or continue distally if reflux was already measured proximally. |
| #7 62–72 s | Ph2 Mid-Thigh Reflux Test | Probe stabilised at mid-thigh (Hunterian canal). Paranà maneuver applied. Doppler shows reflux exceeding 500 ms in the GSV. RP N2→N1 clip marked at posY 0.28. | region=Hunterian posY=0.28 surface=anterior-medial | 2 | Circuit complete — classification confirmed | complete | move | NO | WRONG (1/3) | The AI system's response indicates that the circuit is complete and classification is confirmed, which is incorrect as it fails to direct the probe to the next relevant zone to assess for tributary reflux, instead it should guide the probe distally along the thigh and into the calf to examine the Dodd zone or upper calf. |
| #8 72–82 s | Ph2 Dodd Zone Transition | Probe moves to distal thigh (Dodd perforator zone, just above knee). GSV still visible in fascial compartment. No escape to tributary at this level. Probe preparing to cross knee crease to calf. | region=Dodd posY=0.42 surface=anterior-medial | 2 | Circuit complete — classification confirmed | complete | move | NO | WRONG (1/3) | The AI system's response of 'Circuit complete — classification confirmed' with an action flag of 'complete' is incorrect as it does not direct the probe to continue searching for the escape point in the upper calf, instead suggesting the procedure is finished, which is not the case. |
| #9 82–92 s | Ph3 Upper Calf Entry | Probe moved to upper medial calf just below knee crease. Small GSV oval enters the calf from the thigh. No tributary junction yet at this level. | region=Calf posY=0.61 surface=anterior-medial | 2 | Circuit complete — classification confirmed | complete | move | NO | WRONG (1/3) | The AI system's response of 'Circuit complete — classification confirmed' with an action flag of 'complete' is incorrect as it does not direct the probe to continue scanning distally along the medial calf for the tributary junction, which is the clinical goal at this step. The guidance should instead instruct the probe to move and search for EP N2→N3. |
| #10 92–102 s | Ph3 Mid-Calf Tracking | Probe at mid-calf, anteromedial. GSV smaller. VLM now shows a small N3 structure adjacent to the GSV — possible tributary junction. | region=Calf posY=0.67 surface=anterior-medial | 2 | Circuit complete — classification confirmed | complete | move | NO | WRONG (1/3) | The AI system's response ignores the newly visible N3 tributary structure and fails to instruct the surgeon to assess blood flow into this tributary or mark the EP N2→N3 site, instead prematurely concluding the circuit as complete. |
| #11 102–112 s | Ph3 EP N2→N3 Confirmed | Paranà at mid-calf confirms flow from GSV into the adjacent N3 tributary. Surgeon marks EP N2→N3 at posY 0.68. Q3 answered. Three clips now confirmed. | region=Calf posY=0.68 surface=anterior-medial | 3 | Move proximally toward SPJ junction in popliteal fossa | move | move | YES | WRONG (1/3) | The AI guidance directs the probe proximally toward the SPJ junction, which is incorrect as the clinical intent is to follow the N3 tributary distally along the medial calf to determine where it re-enters the deep system, thereby answering Q4. |
| #12 112–120 s | Ph4 Tributary Tracking | Probe moved to lower medial calf following the N3 tributary distally. Tributary widens slightly as it approaches a perforator site. Re-entry perforator not yet confirmed. | region=Calf posY=0.76 surface=anterior-medial | 3 | Move distally to GSV at medial malleolus (ankle) | move | move | YES | WRONG (1/3) | The AI guidance directs the probe to the GSV at the medial malleolus, which is not the correct structure to track at this step, as the clinical intent is to follow the N3 tributary to its re-entry point into a deep vein, specifically the N1 deep vein. |
| #13 120–130 s | Ph4 Re-entry Perforator Zone | Probe at lower calf near a fascial defect. Tributary appears to dive through the fascia. Diastolic inward flow visible on Paranà release — typical biphasic re-entry perforator pattern. | region=Calf posY=0.80 surface=anterior-medial | 3 | Move medially toward re-entry perforator on medial calf | move | move | YES | WRONG (1/3) | The AI guidance instructs the surgeon to move medially toward the re-entry perforator, which would move the probe away from the correct site where the circuit closes, instead of confirming the inward diastolic flow at the current perforator site and marking it as RP N3→N1. |
| #14 130–138 s | Ph4 RP N3→N1 Confirmed | RP N3→N1 marked at lower calf (posY 0.81). All four circuit clips confirmed: EP N1→N2 + RP N2→N1 + EP N2→N3 + RP N3→N1. Circuit is complete. Elimination test now required. | region=Calf posY=0.81 surface=anterior-medial | 4 | Perform elimination test at current zone | maneuver | maneuver | YES | CORRECT (3/3) | The AI system's guidance to 'Perform elimination test at current zone' with an action flag of 'maneuver' is appropriate, as it directs the surgeon to perform the necessary elimination test to diagnose the Type 3 vs 1+2 issue, and the action flag matches the expected action. |
| #15 138–145 s | Ph5 Maneuver Requested | Probe remains at RP N3→N1 site. Surgeon acknowledges elimination test is needed. No new clips. The system should be directing the surgeon to perform the elimination test. | region=Calf posY=0.81 surface=anterior-medial | 4 | Perform elimination test at current zone | maneuver | maneuver | YES | WRONG (1/3) | The AI system response does not provide specific guidance to perform a compression test at the escape point (EP N2→N3 at mid-calf) to determine whether trunk reflux stops, which is essential to distinguish between Type 3 and Type 1+2 reflux, instead it only instructs to perform an elimination test at the current zone. |
| #16 145–155 s | Ph6 SPJ Approach | Probe repositioned to popliteal fossa (posterior thigh, above knee). Two adjacent ovals visible: popliteal vein (N1, larger) and SSV (N2). Surgeon assessing whether SSV contributes a separate Type 2 circuit. | region=SPJ posY=0.52 surface=posterior | 4 | Perform elimination test at current zone | maneuver | maneuver | YES | WRONG (1/3) | The AI system's guidance to perform an elimination test at the current zone ignores the visible SPJ anatomy and the surgeon's intent to assess the SSV circuit, which is a critical step in evaluating the patient's vascular anatomy. |
| #17 155–162 s | Ph6 SPJ Assessed | Paranà at SPJ: SSV shows no reflux into popliteal vein. SPJ is competent — no separate SSV-driven circuit. Surgeon prepares to return probe to mid-calf for elimination test. | region=SPJ posY=0.50 surface=posterior | 4 | Perform elimination test at current zone | maneuver | maneuver | YES | WRONG (1/3) | The AI system's guidance to perform the elimination test at the current zone is incorrect because the probe is currently at the SPJ and needs to be moved to the mid-calf region to perform the compression test at the EP N2→N3 site. The guidance fails to direct the probe to the correct location. |
| #18 162–168 s | Ph6 Return to Escape Site | Probe returns to medial mid-calf, positioning over the EP N2→N3 site at posY 0.68. This is the compression point for the elimination test. | region=Calf posY=0.68 surface=anterior-medial | 4 | Perform elimination test at current zone | maneuver | maneuver | YES | WRONG (1/3) | The AI system response does not explicitly instruct the surgeon to compress the N3 tributary while observing the GSV Doppler for reflux cessation, which is the critical clinical intent at this step, and the guidance text is too vague. |
| #19 168–175 s | Ph7 Elim Test Compression | Surgeon compresses the N3 tributary at the escape site while observing GSV Doppler at SFJ level (held by assistant). GSV reflux CONTINUES during compression — trunk reflux is independent of the escape. This indicates Type 1+2, NOT Type 3. | region=Calf posY=0.68 surface=anterior-medial | 4 | Perform elimination test at current zone | maneuver | maneuver | YES | PARTIAL (2/3) | The AI system's guidance to 'Perform elimination test at current zone' is partially correct as it acknowledges the ongoing elimination test, but it lacks specificity in instructing the surgeon to record the result of the reflux on the clip, which is the crucial next step for diagnosing Type 1+2 reflux. |
| #20 175–181 s | Ph7 Elim Test Result Recorded | Surgeon records elimTest=Reflux on the EP N2→N3 clip. This is the final diagnostic data point. Type 1+2 classification is now unambiguous. | region=Calf posY=0.68 surface=anterior-medial | 4 | Circuit complete — classification confirmed | complete | complete | YES | CORRECT (3/3) | The AI system's response of 'Circuit complete — classification confirmed' with an action flag of 'complete' accurately reflects the completion of the full clip set for Type 1+2 classification, aligning with the expected action and clinical intent. |
| #21 181–184 s | Ph8 SFJ Return Post-Diagnosis | Probe returns to SFJ for anatomical documentation and skin marking. Circuit is fully classified. Surgeon confirms SFJ as primary ligation site. | region=SFJ posY=0.06 surface=anterior-medial | 4 | Circuit complete — classification confirmed | complete | complete | YES | CORRECT (3/3) | The AI system's response correctly indicates that the circuit is complete and classification is confirmed, aligning with the expected action and ground truth, thus providing accurate guidance to the surgeon. |
| #22 184–187 s | Ph8 Final State Check | Final stable probe position at SFJ. Examination concluded. Type 1+2 diagnosis finalized. Ligation at SFJ and disconnection of the escape tributary to be planned. | region=SFJ posY=0.05 surface=anterior-medial | 4 | Circuit complete — classification confirmed | complete | complete | YES | CORRECT (3/3) | The AI system correctly maintains the 'complete' state with the full clip set and elimination test result, confirming the examination is complete and the circuit is classified as Type 1+2 shunt, which aligns with the expected clinical intent and action flag. |

## 3. Clinical Deep-Dive (per step)

### Step 1: 0–5 s — Phase 1 – SFJ Arrival

Surgeon action: Probe first placed at anteromedial groin crease. B-mode shows two adjacent anechoic ovals: larger CFV (N1) and smaller GSV (N2) on the same depth plane at the junction.
Probe: region=SFJ  posY=0.06  surface=anterior-medial
Clips confirmed: 0
System guidance: "Move proximally toward GSV in saphenous compartment at upper thigh"
Action flag: move  (expected: move)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI guidance directs the probe away from the groin, instructing to move proximally toward the GSV in the saphenous compartment at the upper thigh, which is incorrect as the probe has just arrived at the SFJ and needs to confirm the CFV-GSV relationship and prepare for further maneuvers, not move away from the area of interest.

### Step 2: 5–15 s — Phase 1 – SFJ Orientation

Surgeon action: Probe rotated to longitudinal view then back to transverse. Terminal valve identified just distal to SFJ. Probe centred exactly on the saphenofemoral junction.
Probe: region=SFJ  posY=0.07  surface=anterior-medial
Clips confirmed: 0
System guidance: "Move distally toward GSV in saphenous compartment at upper thigh"
Action flag: move  (expected: move)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI system's guidance to move distally toward the GSV is premature and misses the crucial step of assessing terminal valve competence through Valsalva or Paranà maneuver at the current position, which is essential for EP assessment at SFJ. The correct action should be to instruct the surgeon to perform a maneuver to test valve competence, not to move the probe.

### Step 3: 15–30 s — Phase 1 – SFJ Maneuvers

Surgeon action: Surgeon performs Valsalva and Paranà maneuvers at SFJ. Both tests positive — Doppler shows reversal of flow from CFV into GSV on both maneuvers. SFJ incompetence confirmed.
Probe: region=SFJ  posY=0.06  surface=anterior-medial
Clips confirmed: 0
System guidance: "Move distally toward SPJ junction in popliteal fossa"
Action flag: move  (expected: move)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI system's guidance to move distally toward the SPJ junction in the popliteal fossa is incorrect because it misses the most important clinical need at this step, which is to mark the finding of SFJ incompetence or direct the probe to start mapping the GSV trunk reflux, given that both Valsalva and Paranà maneuvers are positive.

### Step 4: 30–42 s — Phase 1 – EP N1→N2 Marked

Surgeon action: Surgeon marks EP N1→N2 at SFJ (posY 0.06). Clip confirmed. SFJ entry point for shunt circuit now established. Q1 answered. Probe remains at groin for a moment before moving distally.
Probe: region=SFJ  posY=0.06  surface=anterior-medial
Clips confirmed: 1
System guidance: "Move distally toward GSV in upper thigh"
Action flag: move  (expected: move)
Evaluator verdict: CORRECT (3/3)
Evaluator reasoning: The AI system's guidance to move distally toward the GSV in the upper thigh aligns with the clinical intent to track the GSV and assess for trunk reflux, directly addressing the now open question Q2.

### Step 5: 42–52 s — Phase 2 – Proximal Thigh

Surgeon action: Probe moved to proximal anteromedial thigh, just below groin crease. GSV oval visible within the saphenous fascial compartment. No perforator identified at this level.
Probe: region=Upper Thigh  posY=0.14  surface=anterior-medial
Clips confirmed: 1
System guidance: "Move distally toward GSV in saphenous compartment at upper thigh"
Action flag: move  (expected: move)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI guidance instructs the surgeon to move distally toward the GSV, but it fails to mention the application of Paranà (augmentation) to detect trunk reflux or to continue scanning toward the Hunterian zone, which is the correct clinical intent at this step.

### Step 6: 52–62 s — Phase 2 – Hunterian Zone

Surgeon action: Probe at medial mid-thigh (Hunterian canal, proximal). GSV still prominent in fascial compartment. Hunterian perforator junction sought but not yet identified.
Probe: region=Hunterian  posY=0.26  surface=anterior-medial
Clips confirmed: 1
System guidance: "Move medially toward Hunterian perforator on medial proximal thigh"
Action flag: move  (expected: move)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI system's guidance to move medially toward the Hunterian perforator is incorrect because the primary goal at this step is to confirm trunk reflux, not to locate the perforator, and the system should have directed the surgeon to apply Paranà at this level or continue distally if reflux was already measured proximally.

### Step 7: 62–72 s — Phase 2 – Mid-Thigh Reflux Test

Surgeon action: Probe stabilised at mid-thigh (Hunterian canal). Paranà maneuver applied. Doppler shows reflux exceeding 500 ms in the GSV. RP N2→N1 clip marked at posY 0.28.
Probe: region=Hunterian  posY=0.28  surface=anterior-medial
Clips confirmed: 2
System guidance: "Circuit complete — classification confirmed"
Action flag: complete  (expected: move)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI system's response indicates that the circuit is complete and classification is confirmed, which is incorrect as it fails to direct the probe to the next relevant zone to assess for tributary reflux, instead it should guide the probe distally along the thigh and into the calf to examine the Dodd zone or upper calf.

### Step 8: 72–82 s — Phase 2 – Dodd Zone Transition

Surgeon action: Probe moves to distal thigh (Dodd perforator zone, just above knee). GSV still visible in fascial compartment. No escape to tributary at this level. Probe preparing to cross knee crease to calf.
Probe: region=Dodd  posY=0.42  surface=anterior-medial
Clips confirmed: 2
System guidance: "Circuit complete — classification confirmed"
Action flag: complete  (expected: move)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI system's response of 'Circuit complete — classification confirmed' with an action flag of 'complete' is incorrect as it does not direct the probe to continue searching for the escape point in the upper calf, instead suggesting the procedure is finished, which is not the case.

### Step 9: 82–92 s — Phase 3 – Upper Calf Entry

Surgeon action: Probe moved to upper medial calf just below knee crease. Small GSV oval enters the calf from the thigh. No tributary junction yet at this level.
Probe: region=Calf  posY=0.61  surface=anterior-medial
Clips confirmed: 2
System guidance: "Circuit complete — classification confirmed"
Action flag: complete  (expected: move)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI system's response of 'Circuit complete — classification confirmed' with an action flag of 'complete' is incorrect as it does not direct the probe to continue scanning distally along the medial calf for the tributary junction, which is the clinical goal at this step. The guidance should instead instruct the probe to move and search for EP N2→N3.

### Step 10: 92–102 s — Phase 3 – Mid-Calf Tracking

Surgeon action: Probe at mid-calf, anteromedial. GSV smaller. VLM now shows a small N3 structure adjacent to the GSV — possible tributary junction.
Probe: region=Calf  posY=0.67  surface=anterior-medial
Clips confirmed: 2
System guidance: "Circuit complete — classification confirmed"
Action flag: complete  (expected: move)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI system's response ignores the newly visible N3 tributary structure and fails to instruct the surgeon to assess blood flow into this tributary or mark the EP N2→N3 site, instead prematurely concluding the circuit as complete.

### Step 11: 102–112 s — Phase 3 – EP N2→N3 Confirmed

Surgeon action: Paranà at mid-calf confirms flow from GSV into the adjacent N3 tributary. Surgeon marks EP N2→N3 at posY 0.68. Q3 answered. Three clips now confirmed.
Probe: region=Calf  posY=0.68  surface=anterior-medial
Clips confirmed: 3
System guidance: "Move proximally toward SPJ junction in popliteal fossa"
Action flag: move  (expected: move)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI guidance directs the probe proximally toward the SPJ junction, which is incorrect as the clinical intent is to follow the N3 tributary distally along the medial calf to determine where it re-enters the deep system, thereby answering Q4.

### Step 12: 112–120 s — Phase 4 – Tributary Tracking

Surgeon action: Probe moved to lower medial calf following the N3 tributary distally. Tributary widens slightly as it approaches a perforator site. Re-entry perforator not yet confirmed.
Probe: region=Calf  posY=0.76  surface=anterior-medial
Clips confirmed: 3
System guidance: "Move distally to GSV at medial malleolus (ankle)"
Action flag: move  (expected: move)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI guidance directs the probe to the GSV at the medial malleolus, which is not the correct structure to track at this step, as the clinical intent is to follow the N3 tributary to its re-entry point into a deep vein, specifically the N1 deep vein.

### Step 13: 120–130 s — Phase 4 – Re-entry Perforator Zone

Surgeon action: Probe at lower calf near a fascial defect. Tributary appears to dive through the fascia. Diastolic inward flow visible on Paranà release — typical biphasic re-entry perforator pattern.
Probe: region=Calf  posY=0.80  surface=anterior-medial
Clips confirmed: 3
System guidance: "Move medially toward re-entry perforator on medial calf"
Action flag: move  (expected: move)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI guidance instructs the surgeon to move medially toward the re-entry perforator, which would move the probe away from the correct site where the circuit closes, instead of confirming the inward diastolic flow at the current perforator site and marking it as RP N3→N1.

### Step 14: 130–138 s — Phase 4 – RP N3→N1 Confirmed

Surgeon action: RP N3→N1 marked at lower calf (posY 0.81). All four circuit clips confirmed: EP N1→N2 + RP N2→N1 + EP N2→N3 + RP N3→N1. Circuit is complete. Elimination test now required.
Probe: region=Calf  posY=0.81  surface=anterior-medial
Clips confirmed: 4
System guidance: "Perform elimination test at current zone"
Action flag: maneuver  (expected: maneuver)
Evaluator verdict: CORRECT (3/3)
Evaluator reasoning: The AI system's guidance to 'Perform elimination test at current zone' with an action flag of 'maneuver' is appropriate, as it directs the surgeon to perform the necessary elimination test to diagnose the Type 3 vs 1+2 issue, and the action flag matches the expected action.

### Step 15: 138–145 s — Phase 5 – Maneuver Requested

Surgeon action: Probe remains at RP N3→N1 site. Surgeon acknowledges elimination test is needed. No new clips. The system should be directing the surgeon to perform the elimination test.
Probe: region=Calf  posY=0.81  surface=anterior-medial
Clips confirmed: 4
System guidance: "Perform elimination test at current zone"
Action flag: maneuver  (expected: maneuver)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI system response does not provide specific guidance to perform a compression test at the escape point (EP N2→N3 at mid-calf) to determine whether trunk reflux stops, which is essential to distinguish between Type 3 and Type 1+2 reflux, instead it only instructs to perform an elimination test at the current zone.

### Step 16: 145–155 s — Phase 6 – SPJ Approach

Surgeon action: Probe repositioned to popliteal fossa (posterior thigh, above knee). Two adjacent ovals visible: popliteal vein (N1, larger) and SSV (N2). Surgeon assessing whether SSV contributes a separate Type 2 circuit.
Probe: region=SPJ  posY=0.52  surface=posterior
Clips confirmed: 4
System guidance: "Perform elimination test at current zone"
Action flag: maneuver  (expected: maneuver)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI system's guidance to perform an elimination test at the current zone ignores the visible SPJ anatomy and the surgeon's intent to assess the SSV circuit, which is a critical step in evaluating the patient's vascular anatomy.

### Step 17: 155–162 s — Phase 6 – SPJ Assessed

Surgeon action: Paranà at SPJ: SSV shows no reflux into popliteal vein. SPJ is competent — no separate SSV-driven circuit. Surgeon prepares to return probe to mid-calf for elimination test.
Probe: region=SPJ  posY=0.50  surface=posterior
Clips confirmed: 4
System guidance: "Perform elimination test at current zone"
Action flag: maneuver  (expected: maneuver)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI system's guidance to perform the elimination test at the current zone is incorrect because the probe is currently at the SPJ and needs to be moved to the mid-calf region to perform the compression test at the EP N2→N3 site. The guidance fails to direct the probe to the correct location.

### Step 18: 162–168 s — Phase 6 – Return to Escape Site

Surgeon action: Probe returns to medial mid-calf, positioning over the EP N2→N3 site at posY 0.68. This is the compression point for the elimination test.
Probe: region=Calf  posY=0.68  surface=anterior-medial
Clips confirmed: 4
System guidance: "Perform elimination test at current zone"
Action flag: maneuver  (expected: maneuver)
Evaluator verdict: WRONG (1/3)
Evaluator reasoning: The AI system response does not explicitly instruct the surgeon to compress the N3 tributary while observing the GSV Doppler for reflux cessation, which is the critical clinical intent at this step, and the guidance text is too vague.

### Step 19: 168–175 s — Phase 7 – Elim Test Compression

Surgeon action: Surgeon compresses the N3 tributary at the escape site while observing GSV Doppler at SFJ level (held by assistant). GSV reflux CONTINUES during compression — trunk reflux is independent of the escape. This indicates Type 1+2, NOT Type 3.
Probe: region=Calf  posY=0.68  surface=anterior-medial
Clips confirmed: 4
System guidance: "Perform elimination test at current zone"
Action flag: maneuver  (expected: maneuver)
Evaluator verdict: PARTIAL (2/3)
Evaluator reasoning: The AI system's guidance to 'Perform elimination test at current zone' is partially correct as it acknowledges the ongoing elimination test, but it lacks specificity in instructing the surgeon to record the result of the reflux on the clip, which is the crucial next step for diagnosing Type 1+2 reflux.

### Step 20: 175–181 s — Phase 7 – Elim Test Result Recorded

Surgeon action: Surgeon records elimTest=Reflux on the EP N2→N3 clip. This is the final diagnostic data point. Type 1+2 classification is now unambiguous.
Probe: region=Calf  posY=0.68  surface=anterior-medial
Clips confirmed: 4
System guidance: "Circuit complete — classification confirmed"
Action flag: complete  (expected: complete)
Evaluator verdict: CORRECT (3/3)
Evaluator reasoning: The AI system's response of 'Circuit complete — classification confirmed' with an action flag of 'complete' accurately reflects the completion of the full clip set for Type 1+2 classification, aligning with the expected action and clinical intent.

### Step 21: 181–184 s — Phase 8 – SFJ Return Post-Diagnosis

Surgeon action: Probe returns to SFJ for anatomical documentation and skin marking. Circuit is fully classified. Surgeon confirms SFJ as primary ligation site.
Probe: region=SFJ  posY=0.06  surface=anterior-medial
Clips confirmed: 4
System guidance: "Circuit complete — classification confirmed"
Action flag: complete  (expected: complete)
Evaluator verdict: CORRECT (3/3)
Evaluator reasoning: The AI system's response correctly indicates that the circuit is complete and classification is confirmed, aligning with the expected action and ground truth, thus providing accurate guidance to the surgeon.

### Step 22: 184–187 s — Phase 8 – Final State Check

Surgeon action: Final stable probe position at SFJ. Examination concluded. Type 1+2 diagnosis finalized. Ligation at SFJ and disconnection of the escape tributary to be planned.
Probe: region=SFJ  posY=0.05  surface=anterior-medial
Clips confirmed: 4
System guidance: "Circuit complete — classification confirmed"
Action flag: complete  (expected: complete)
Evaluator verdict: CORRECT (3/3)
Evaluator reasoning: The AI system correctly maintains the 'complete' state with the full clip set and elimination test result, confirming the examination is complete and the circuit is classified as Type 1+2 shunt, which aligns with the expected clinical intent and action flag.

## 4. Phase-Level Summary

| Phase | Steps | CORRECT | PARTIAL/WRONG | Action OK |
| --- | --- | --- | --- | --- |
| Phase 1 | 4 | 1/4 | 3 | 4/4 |
| Phase 2 | 4 | 0/4 | 4 | 2/4 |
| Phase 3 | 3 | 0/3 | 3 | 1/3 |
| Phase 4 | 3 | 1/3 | 2 | 3/3 |
| Phase 5 | 1 | 0/1 | 1 | 1/1 |
| Phase 6 | 3 | 0/3 | 3 | 3/3 |
| Phase 7 | 2 | 1/2 | 1 | 2/2 |
| Phase 8 | 2 | 2/2 | 0 | 2/2 |